Router#show running-config

Building configuration...

Current configuration : 1700 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Router

!

!

!

!

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

!

ip dhcp pool HR

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

ip dhcp pool IT

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

ip dhcp pool SALES

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

ip dhcp pool SERVERS

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

dns-server 8.8.8.8

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX15249VA4-

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.10

description HR Gateway

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

!

interface GigabitEthernet0/0.20

description IT Gateway

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

!

interface GigabitEthernet0/0.30

description SALES Gateway

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

!

interface GigabitEthernet0/0.40

description SERVER Gateway

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end